🏄‍♂️ BRemote V2 - Custom Tow Boogie Edition

Welcome to the custom firmware fork for the BRemote V2. This firmware is heavily optimized for electric Tow Boogies and eFoils, featuring an integrated VESC/GPS data logger, an upgraded mobile Web Console, and custom telemetry features.

📡 Transmitter (TX) Features

Standard Features:

- LoRa Radio Link: Long-range, rock-solid 868MHz or 915MHz connection.

- Dynamic Power Cap: Adjust your max motor power on-the-fly using the thumbstick (Throttle Mode 2).

- Live OLED Telemetry: View VESC battery, remote battery, motor temperature, and power (Watts).

- Failsafe Protection: Motor cuts out automatically if the signal drops for 1 second.

- Trigger Expo & Deadzones: Fully customizable throttle curves for buttery smooth starts.

<span style="color: #10b981; font-weight: bold;">✨ CUSTOM TX ADDITIONS:</span>

- <span style="color: #10b981;"><b>Miles Per Hour (MPH) Display:</b></span> Added new speed_src options (4 & 5) to display speed natively in MPH rather than just km/h or knots.

- <span style="color: #10b981;"><b>Watchdog Stability Patch:</b></span> Eliminated the 1.5-second reboot loop bug that occurred when unlocking the remote.

- <span style="color: #10b981;"><b>V3 Config Studio:</b></span> A brand new, standalone HTML tool to easily generate, compare, and convert your configurations between JSON (Mobile) and Base64 (Serial Console).

🎛️ Receiver (RX) Features

Standard Features:

- VESC UART Integration: Reads live RPM, Voltage, Amps, and Temperatures straight from the motor controller.

- Servo Steering: Converts thumbstick inputs into PWM signals for your boogie's rudder servo.

- Water Ingress Detection: Triggers an emergency "E71" alarm on your remote if water bridges the sensor wires inside the boogie hatch.

- GPS Integration: Ready for TinyGPS++ modules (like the BN-220) for speed tracking and Follow-Me autonomous modes.

<span style="color: #10b981; font-weight: bold;">✨ CUSTOM RX ADDITIONS:</span>

- <span style="color: #10b981;"><b>Integrated Hardware Logger:</b></span> Saves high-resolution VESC data (RPM, Amps, Watts) and GPS coordinates directly to the RX flash memory at 1Hz.

- <span style="color: #10b981;"><b>US Date Stamps:</b></span> Log files are now beautifully and cleanly formatted using UTC time as MMDDYY_HHMMSS.log (e.g., 041726_123045.log).

- <span style="color: #10b981;"><b>Upgraded Mobile Web Console:</b></span>

  - Dirty State Highlighting: Input boxes turn dark red when changed so you don't forget to save.

  - Toast Notifications: Green pop-up banners confirm successful saves and loads.

  - Over-The-Air Log Downloads: A brand new "Manage Logs" menu allows you to download your CSV data logs directly to your phone via WiFi.

📊 The Data Logger Guide (How to use it)

Your Receiver is now equipped with a powerful background logger to help you test propeller efficiency.

🔘 The AUX Button & LED Sequences

You do not need your phone or computer to log data. Simply use the physical AUX button inside your boogie hatch.

- Wait for GPS Lock: Turn on the boogie. The logger will not start until the GPS has a solid satellite lock (to ensure accurate timestamps).

- Start Recording: Press the AUX button once.

  - LED Confirmation: The blue LED will flash 5 times rapidly.

  - Status: The LED will then remain solidly ON while data is recording.

- Stop Recording: Press the AUX button again.

  - LED Confirmation: The blue LED will flash 2 times slowly.

  - Status: The LED will turn OFF, and your .csv file is safely closed and saved.

⚠️ IMPORTANT: The "Brownout" Warning & WiFi Auto-Kill

Writing data to the internal flash memory draws a massive spike of electrical current (up to 800mA).

- The Auto-Kill Feature: To save power and CPU cycles, your Receiver's WiFi is programmed to turn off automatically the exact second your TX Remote connects to it, OR after 60 seconds of inactivity.

- The Warning: If you are bench-testing the boogie using a weak laptop USB port, and you try to stop/save a log while the WiFi network is still actively broadcasting, the combined power draw can cause a "Brownout" (the ESP32 will reboot to protect itself).

- Best Practice: Always wait for the WiFi network to disappear (either by turning on your handheld remote, or waiting 60 seconds) before you start or stop your logs. (Note: When the boogie is fully assembled and powered by the VESC's strong 5V BEC, this is rarely an issue, but it is best practice!)

🗺️ Follow-Me Visualizer Tool

To help you tune your autonomous "Follow-Me" settings (min_dist, smoothing_band, zone_angles, etc.), use the HTML Visualizer tool below.

To use it, simply save these three blocks of code into a folder on your computer and double-click